DAY 2 TRAINER SCRIPT Title: Physics, System Overview & Pre-Job Inspection

Overview This Trainer Script provides a full, verbatim script for delivering Day 2 of the High Pressure Water Jetting training module. It follows the approved Day 2 slide deck exactly, ensuring smooth, engaging delivery in the same professional style as the ANABEEB NEBOSH Confined Space course and the approved Day 1 script.

The script uses complete sentences in simple English. Italics are cues for the trainer (interactions, pauses, pointing to images). Speak clearly, repeat key terms, and relate everything to real oil & gas site work.

Total Day 2 Duration: 6–7 hours (including two 15-min breaks, lunch/prayer, practical inspection activity, and Q&A). Resources required:

- Projector & approved Day 2 slide deck

- Training rig / pump, hoses, fittings, nozzles, whip checks, shrouds (for practical)

- Printed pre-job inspection checklists (one per team)

- Pin gauges, thread sealant, colour-code samples

- Day 1 Medical Alert Card review list

Slide 1: Title Slide Good morning/afternoon, everyone. Welcome back to Day 2 of the Anabeeb High Pressure Water Jetting Basic Jetter & LTC Operator Certification program. Today we cover Physics, System Overview and Pre-Job Inspection.

As shown in the image, we are looking at the complete high-pressure water jetting system. Understanding how every part works and how to inspect it properly is the foundation of safe operation.

Let’s start with a quick review of Day 1. Please show me your Medical Alert Card — you must carry it at all times. [Pause while trainees show cards]. Excellent. Any questions from yesterday before we begin? [Pause for responses]. Let’s move into today’s housekeeping.

Slide 2: Housekeeping & Day 2 Expectations Before we go further, let’s cover housekeeping and expectations for Day 2. We have a full day with scheduled breaks, prayer time and lunch. Dress every day as if you are going to the job site — proper safety boots and no shorts.

Today is 50 % theory and 50 % practical inspection work. Full participation is expected and questions are always welcome. Safety has no dumb questions.

Any questions on the housekeeping? [Pause for responses]. Great — let’s look at exactly what you will learn today.

Slide 3: Day 2 Learning Objectives By the end of today you will be able to:

• Understand how the high-pressure water jetting system works • Explain the difference between pressure and flow • Identify the different pressure relief control styles • Recognise barricaded zones and safe distances • Perform a complete pre-job inspection of the pump, hoses, fittings and nozzles

These skills are essential for every job you will do on site. Which objective do you think will be most useful for your daily work? [Pause for responses]. Let’s start with the basic principle of how the entire system operates.

Slide 4: How Does the Waterjetting System Work? How does the waterjetting system actually work? The pump only moves or displaces water — it does not create pressure.

Pressure is created only when the flow is restricted at the end of the hose by the nozzle. That is why the nozzle operator must always have control of the pressure relief.

The most important principle on every job is this: know exactly where the water is going to go at all times. [Point to the key sentence]. Has anyone seen what happens when the operator loses control of where the water goes? [Pause for responses]. Let’s now look at pressure versus flow in more detail.

Slide 5: Pressure and Flow Let’s talk about pressure versus flow. Flow is the volume of water the pump moves, measured in litres per minute or gallons per minute. Pressure is the force created when that flow is restricted by the nozzle.

Higher pressure combined with a smaller nozzle orifice gives greater cutting power, but it also creates higher reaction force on the operator.

Important point to remember: the pump only displaces water — the nozzle is what creates the pressure. [Point to the important note]. Any questions so far? [Pause for responses]. Now let’s look at the different ways we control and relieve that pressure.

Slide 6: Pressure Relief Control Styles There are three common pressure relief control styles you will see on site:

• Standard dump style • Pneumatic style • Dry-shut or electrical dump style

The critical rule is that the nozzle operator must always have positive control of the pressure relief method. Different styles may add a few extra steps, but they never change the overall safety sequence.

[Point to each style]. Which style have you seen most often on your previous jobs? [Pause for responses]. Let’s now look at how we protect everyone around the work area.

Slide 7: Barricaded Zones Barricaded zones are a key safety control. Temporary barricades must be erected to keep unauthorised people out of the hazard area.

The minimum safe distance follows industry best practices — typically 6 metres in all directions. We use red barricade tape with clear signs that say “DANGER – HIGH PRESSURE WATER JETTING IN PROGRESS”.

If the full 6-metre distance cannot be achieved, we use additional shrouds or physical shielding. Only authorised personnel are allowed inside the barricaded area. [Point to the signage text]. Why do you think we use red tape specifically? [Pause for responses]. Now we move into the main part of today — the pre-job inspection.

Slide 8: Pre-Job Inspection Overview The high-pressure system must be inspected before every single use. Today we will cover every item on the checklist.

We will look at: • The high-pressure pump • Adapters and fittings • Hoses and hose restraints • Nozzles • Whip checks and shrouds

This inspection is not optional — it is mandatory. [Point to the list]. Let’s start with the pump.

Slide 9: High-Pressure Pump Inspection For the high-pressure pump we check the following:

• The pump is level and wheels are chocked • All guards and shrouds are in place — plungers, belts and fans • The pump is grounded to valid earth • Rupture disc housing is tagged and valid • Calibrated pressure gauge is fitted • Fire extinguisher is accessible and in date • Filter bag is installed and clean

[Point to each bullet]. These checks protect both the equipment and the people around it. Any questions on the pump inspection? [Pause for responses]. Next are the adapters and fittings.

Slide 10: Adapters and Fittings Adapters and fittings are critical. Every fitting must be permanently marked with its maximum pressure rating and must match the pump capability.

We always check for proper thread engagement and clean sealing surfaces. Never use mismatched or damaged fittings.

Important note: 10k/15k and 20k fittings can look similar, but they have different thread and seat designs. Using the wrong one can cause a catastrophic failure. [Point to the critical note]. Let’s move on to the hoses.

Slide 11: High-Pressure Hoses – Visual Inspection High-pressure hoses must be visually inspected every time before use. Look for:

• Exposed wire braids • Bubbling or damage to the outer jacket • Kinks, cuts or crushed sections • Damaged couplings or crimps • Exceeded bend radius

Any hose that fails any of these points must be removed from service immediately. [Point to the list]. Why is visual inspection so important? [Pause for responses]. Let’s look at colour coding and service life next.

Slide 12: Hose Colour Coding & Service Life Hoses must be colour-coded to match or exceed the system operating pressure.

Maximum service life from the date of manufacture is: – Supply hoses: 4 years – Lance and whip hoses: 2 years

Every hose must be clearly marked with its in-service date. [Point to the life limits]. This prevents us from using hoses that have become weak over time.

Slide 13: Hose Restraints (Whip Checks) Hose restraints, or whip checks, must be installed across every hose connection. They must be rated for the hose size and pressure.

Exceptions are only for connections that travel inside pipe or through a hose-feeding device. Hose shrouds are required at every gun or lance connection and must be a minimum of 6 feet or 2 metres long.

[Point to the exceptions and shroud requirement]. These devices stop a hose from whipping if a connection fails.

Slide 14: Nozzles Nozzles are the heart of the system. Every nozzle must be:

• Rated for the hose size and system pressure • Made of stainless steel • Clean and undamaged (we use pin gauges to check orifices) • With threads and sealing surfaces in good condition

We use straight tips, fan tips or rotating nozzles depending on the job. [Point to the list]. Let’s now cover cold weather operation.

Slide 15: Cold Weather Operation In cold weather we must never start a frozen system. Always use an approved winterising solution and circulate it through all lines, including dump and bypass circuits.

Monitor for ice bullets or split components. Any hose that has experienced a hard freeze should be removed from service.

[Point to the key rules]. This prevents expensive damage and dangerous projectiles.

Slide 16: Full Pre-Job Inspection Checklist Here is the complete pre-job inspection checklist we use on every job:

• Pump level and chocked • All guards in place • Hoses inspected and colour-coded • Whip checks and shrouds fitted • Nozzles clean and rated • Fittings matched to pressure • System leak-tested at operating pressure

This checklist must be completed and signed before every job starts. [Point to the list]. Now it is time to practise what we have learned.

Slide 17: Practical Activity – Pre-Job Inspection Practical time. Please move with me to the training rig and equipment area.

Work in teams of 2–3. Using the checklist, perform a complete pre-job inspection of the pump, hoses, fittings and nozzles. I will observe each team and sign off every item.

You must achieve 100 % competency before we finish today. [Point to the slide]. Let’s begin. [Conduct the practical — allow 45–60 minutes, rotate teams, provide coaching].

Slide 18: Day 2 Summary Day 2 Summary — Key Takeaways:

• The pump only displaces water — the nozzle creates the pressure • The nozzle operator must always control pressure relief • Full pre-job inspection is mandatory before every use • You have now practised a complete system inspection

Tomorrow on Day 3 we cover PPE plus Job Safety Analysis and the Pre-Job Checklist.

Any final questions on today’s material? [Pause for responses].

Slide 19: Thank You & Contact Thank You

You have completed Day 2 of the Anabeeb High Pressure Water Jetting program. Great work on the practical inspections today.

If you have any further questions, please feel free to reach out.

Arabian Pipeline & Services Co. Ltd. (ANABEEB) P.O. Box 234, Dammam | Tel: +966 13 xxx xxxx | Email: training@anabeeb.com www.anabeeb.com

Any final questions before we close? [Pause for responses]. Safe travels home and see you tomorrow dressed for site!