Day 2 Physics, System Overview & Pre-Job Inspection

ANABEEB HSE & Training Trainer: _____________________ Date: _______________ Location: Dammam

(Image: Cut-away view of high-pressure pump and hose system)

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 2: Housekeeping & Day 2 Expectations END-TO-END SOLUTIONS

HOUSEKEEPING & DAY 2 EXPECTATIONS

• Review of Day 1 Medical Alert Cards • Dress as if going to site (proper boots, no shorts) • Full participation & questions encouraged • Today is 50 % theory + 50 % practical inspection • Safety has no dumb questions

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 3: Day 2 Learning Objectives END-TO-END SOLUTIONS

DAY 2 LEARNING OBJECTIVES

• Understand how the high-pressure water jetting system works • Explain pressure versus flow • Identify different pressure relief control styles • Recognise barricaded zones and safe distances • Perform a complete pre-job inspection of pump, hoses, fittings and nozzles

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 4: How Does the Waterjetting System Work? END-TO-END SOLUTIONS

HOW DOES THE WATERJETTING SYSTEM WORK?

• The pump moves (displaces) water — it does not create pressure • Pressure is created only by the restriction at the end of the hose (the nozzle) • The nozzle operator must always have control of the pressure relief

Key principle: Know exactly where the water is going to go at all times

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 5: Pressure and Flow END-TO-END SOLUTIONS

PRESSURE VERSUS FLOW

• Flow = volume of water moved by the pump (litres per minute or GPM) • Pressure = force created when flow is restricted by the nozzle • Higher pressure + smaller nozzle orifice = greater cutting power and higher reaction force

Important: The pump only displaces water — the nozzle creates the pressure

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 6: Pressure Relief Control Styles (Revised) END-TO-END SOLUTIONS

PRESSURE RELIEF CONTROL STYLES

• Standard dump style • Pneumatic style • Dry-shut / electrical dump style

Critical rule: The nozzle operator must always have positive control of the pressure relief method. Different styles may add extra steps but never change the safety sequence.

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 7: Barricaded Zones END-TO-END SOLUTIONS

BARRICADED ZONES

• Temporary barricades required to restrict access to the hazard area • Minimum safe distance per industry best practices (typically 6 metres in all directions) • Use red barricade tape with clear “DANGER – HIGH PRESSURE WATER JETTING IN PROGRESS” signage • If 6 m distance cannot be achieved, use additional shrouds or physical shielding

Only authorised personnel allowed inside the barricaded area

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 8: Pre-Job Inspection Overview END-TO-END SOLUTIONS

PRE-JOB INSPECTION OVERVIEW

The high-pressure system must be inspected before every use. Today we will cover: • High-pressure pump • Adapters and fittings • Hoses and hose restraints • Nozzles • Whip checks and shrouds

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 9: High-Pressure Pump Inspection END-TO-END SOLUTIONS

HIGH-PRESSURE PUMP INSPECTION

• Pump is level and wheels chocked • All guards and shrouds in place (plungers, belts, fans) • Grounded to valid earth • Rupture disc housing tagged and valid • Calibrated pressure gauge fitted • Fire extinguisher accessible and in date • Filter bag installed and clean

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 10: Adapters and Fittings END-TO-END SOLUTIONS

ADAPTERS AND FITTINGS

• All fittings must be permanently marked with maximum pressure rating • Match fitting rating to pump capability • Check for proper thread engagement and sealing surfaces • Never use mismatched or damaged fittings

Critical: 10k / 15k and 20k fittings look similar but have different thread and seat designs

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 11: High-Pressure Hoses – Visual Inspection END-TO-END SOLUTIONS

HIGH-PRESSURE HOSES – VISUAL INSPECTION

Inspect hoses for: • Exposed wire braids • Bubbling or damage to outer jacket • Kinks, cuts or crushed sections • Damaged couplings or crimps • Exceeded bend radius

Any damaged hose must be removed from service immediately

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 12: Hose Colour Coding & Service Life END-TO-END SOLUTIONS

HOSE COLOUR CODING & SERVICE LIFE

• Hoses must be colour-coded to match or exceed the system operating pressure • Maximum service life (from date of manufacture): – Supply hoses: 4 years – Lance / whip hoses: 2 years • Hoses must be marked with in-service date

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 13: Hose Restraints (Whip Checks) END-TO-END SOLUTIONS

HOSE RESTRAINTS (WHIP CHECKS)

• Whip checks must be installed across every hose connection • Rated for the hose size and pressure • Exceptions: connections inside pipe or through hose-feeding devices • Hose shrouds required at gun/lance connections (minimum 6 ft / 2 m)

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 14: Nozzles END-TO-END SOLUTIONS

NOZZLES

• Must be rated for hose size and system pressure • Made of stainless steel • Orifices clean and undamaged (use pin gauges to check) • Threads and sealing surfaces in good condition • Straight, fan or rotating nozzles each have different uses

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 15: Cold Weather Operation END-TO-END SOLUTIONS

COLD WEATHER OPERATION

• Never start a frozen system • Use approved winterising solution and circulate through all lines • Monitor for ice bullets or split components • Hoses that have experienced a hard freeze should be removed from service

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 16: Full Pre-Job Inspection Checklist END-TO-END SOLUTIONS

FULL PRE-JOB INSPECTION CHECKLIST

• Pump level & chocked • All guards in place • Hoses inspected & colour-coded • Whip checks and shrouds fitted • Nozzles clean and rated • Fittings matched to pressure • System leak-tested at operating pressure

This checklist must be completed and signed before every job

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 17: Practical Activity – Pre-Job Inspection END-TO-END SOLUTIONS

PRACTICAL ACTIVITY – PRE-JOB INSPECTION

• Move to the training rig / equipment area • Work in teams of 2–3 • Complete the full pre-job inspection checklist • Trainer will observe and sign off each item

100 % competency required

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 18: Day 2 Summary END-TO-END SOLUTIONS

DAY 2 SUMMARY

Key Takeaways: • The pump displaces water — the nozzle creates pressure • The operator must always control pressure relief • Full pre-job inspection is mandatory before every use • You have now practised a complete system inspection

Tomorrow (Day 3): PPE + Job Safety Analysis & Pre-Job Checklist

Footer: High Pressure Water Jetting (HPWJ)

SLIDE 19: Thank You & Contact END-TO-END SOLUTIONS

THANK YOU

Arabian Pipeline & Services Co. Ltd. (ANABEEB) P.O. Box 234, Dammam | Tel: +966 13 xxx xxxx | Email: training@anabeeb.com www.anabeeb.com

Footer: High Pressure Water Jetting (HPWJ)