Anabeeb HPWJ Fundamentals

Written Examination — Marking Key (ASSESSOR COPY ONLY)

ATC-HPWJ-OP-001   |   Total marks: 70   |   Pass mark: 56 (80%)

Control: This document must not be visible to candidates at any time. Store separately from exam papers. Version-control any question changes together with this key.

# Exam Blueprint Compliance

Core safety rules (team size, pressure ceiling, rupture disc, exclusion, stop-work, reaction-force limits, hose/lance/AWD rules): approximately 40 of 70 marks (≈57%) — exceeds the 30% minimum.

Injection-injury recognition and emergency response (Q5–Q8, Q34, Q35, Q39, Q44): 14 of 70 marks (20%) — meets the 20% minimum.

# Section A — Correct Letters

Q1: C

Q2: B

Q3: B

Q4: B

Q5: D

Q6: B

Q7: C

Q8: B

Q9: B

Q10: C

Q11: B

Q12: C

Q13: C

Q14: D

Q15: B

Q16: C

Q17: B

Q18: C

Q19: B

Q20: C

Q21: B

Q22: C

Q23: C

Q24: C

Q25: B

Q26: C

Q27: C

Q28: B

Q29: B

Q30: C

# Section B — Model Answers

Q31: Lance operator; pump operator; dedicated emergency-stop person. (All three for full marks; 1 mark for two correct.)

Q32: Force (lbf) ≈ 0.052 × flow (GPM) × √pressure (psi). (Formula structure correct = 2; missing square root = 0.)

Q33: Dump pressure and stop immediately (1); because no change of state is safe without confirmed communication / restart only after communication is re-established (1).

Q34: Any two of: the entry wound often looks minor while internal damage is severe; injected fluid spreads and damage worsens rapidly; delay in correct treatment drives poor outcomes. (1 each.)

Q35: A waterproof card explaining the injection-injury mechanism for medical responders (1); carried immediately accessible at all times on every job (1).

Q36: Simultaneous operations — jetting while other work happens nearby (1); example, e.g. crane lifting overhead, hot work near the jetting area, another crew sharing the ground (1).

Q37: Any two of: skipped/inadequate pre-use inspection; loss of control of a flexible lance; poor coordination between simultaneous activities (SIMOPS). (1 each.)

Q38: PPE is rated by pressure class (1); equipment rated for a lower class does not protect at higher pressures — e.g. a 10,000 psi suit does not protect on a 40,000 psi job (1).

Q39: Dump pressure / stop the jet (1); because no response is safe, and no rescue possible, until the energy source is removed (1).

Q40: It warns that the nozzle tip is about to exit the tube (1); the exit moment is the highest back-out / loss-of-control risk (1).

# Section C — Model Answers

Q41: Force ≈ 0.052 × 8 × 110 ≈ 45.8 lbf (1) ≈ 204 N (1). Passes the 250 N absolute limit (1). Must still check: under one third of the specific operator's body weight AND barrel geometry keeps the jet off the operator's own feet/legs (1).

Q42: Force ≈ 0.233 × 40 × 34.6 ≈ 322 N (2). Exceeds the 250 N absolute limit (1). Hand-held operation is not permitted as configured — the job needs mechanical support, a different nozzle, reduced parameters, or automation (1).

Q43: Stop work / dump pressure (1). Do not attempt to negotiate or resolve the work-area conflict yourself (1). Raise it with the supervisor immediately (1). Work restarts only after supervisors coordinate — daily SIMOPS coordination, agreed zoning, and marked boundaries re-established (1).

Q44: Dump pressure and stop work first (1). Treat as a medical emergency regardless of appearance / never wait and see (1). Deliver the script: 'This is a high-pressure water injection injury' + location, and get emergency help (1). Present the Medical Alert Card to responders and do not allow him to continue working (1).

Q45: Refuse to operate — a flexible lance without a correctly installed and verified AWD is not permitted, no exceptions (2). Call stop-work — the call is absolute and cannot be overridden for schedule (1). Escalate beyond the supervisor if pressed (HSE / management), citing that this exact failure mode has killed an operator (1).

## Marking guidance

Section C answers need not match the model wording — award marks for each distinct correct element as bracketed. For calculations, accept small rounding differences (±5%). An answer that reaches the wrong pass/fail decision from a correct calculation scores the calculation marks but not the decision mark. An answer that skips the calculation but states the correct decision with reasoning scores the decision mark only.