Anabeeb HPWJ Fundamentals

Written Examination — ATC-HPWJ-OP-001

Time allowed: 60 minutes   |   Total marks: 70   |   Pass mark: 56 (80%)

Candidate name: ______________________________    Date: ______________    Candidate ID: ______________

Instructions: Individual work. Closed notes. Answer ALL questions. For Section A, circle ONE letter per question. For Sections B and C, write your answer on the lines provided. Read scenario questions twice before answering. Calculators are permitted for Section C.

# Section A — Multiple Choice (30 questions × 1 mark = 30 marks)

1. What is the absolute maximum working pressure for any HPWJ activity?  (1 mark)

A)  25,000 psi

B)  30,000 psi

C)  40,000 psi

D)  50,000 psi

2. What is the minimum team size for any HPWJ task?  (1 mark)

A)  Two competent people

B)  Three competent people, one dedicated solely to the emergency stop

C)  Four competent people

D)  One operator plus a supervisor on call

3. The rupture disc must be rated no higher than:  (1 mark)

A)  The pump's maximum pressure

B)  1.2 times the weakest-rated part in the system

C)  1.5 times the working pressure

D)  2 times the nozzle rating

4. How close may an unauthorised person come to running HPWJ equipment?  (1 mark)

A)  No closer than 5 metres

B)  No closer than 10 metres

C)  No closer than 15 metres

D)  Anywhere outside the spray area

5. At 40,000 psi, the jet exit velocity is approximately:  (1 mark)

A)  150 m/s

B)  340 m/s

C)  500 m/s

D)  743 m/s

6. Why is a small-looking entry wound from a water jet still dangerous?  (1 mark)

A)  It is usually not dangerous if small

B)  Internal damage is frequently severe and worsens rapidly

C)  Only dirty water causes harm

D)  It only matters above 20,000 psi

7. The correct approach to ANY suspected injection injury is:  (1 mark)

A)  Monitor it for 24 hours

B)  Wash it and apply a dressing

C)  Treat it as a medical emergency until proven otherwise

D)  Only act if pain increases

8. The biggest driver of poor outcomes in injection injuries is:  (1 mark)

A)  The size of the entry wound

B)  Delay in correct treatment

C)  The water temperature

D)  The operator's age

9. The three additive reaction-force controls are: force under 250 N, AND …  (1 mark)

A)  …nozzle under 1.6 mm and lance over 1.2 m

B)  …under one third of the operator's body weight and geometry keeping the jet off the operator's own feet/legs

C)  …under half body weight and two-hand grip

D)  …under 10,000 psi and a whip check fitted

10. If any ONE of the three reaction-force checks fails:  (1 mark)

A)  Proceed with extra care

B)  Add a second operator on the lance

C)  Handheld operation is not permitted — use mechanical support, a longer barrel, a different nozzle, or automation

D)  Reduce the job to 30-minute shifts

11. A full documented pre-use inspection is required:  (1 mark)

A)  Weekly

B)  Before every use and every shift

C)  Only after maintenance

D)  Only for UHP work

12. The relationship between the pre-use inspection and the work permit is:  (1 mark)

A)  They are independent

B)  The permit can be issued while inspection is in progress

C)  No permit is issued without a completed and verified pre-use inspection checklist

D)  The permit replaces the inspection

13. Who can call stop-work on an HPWJ job?  (1 mark)

A)  Only the supervisor

B)  Only the HSE representative

C)  Any team member

D)  Only the client representative

14. A stop-work call can be overridden when:  (1 mark)

A)  The schedule is critical

B)  The supervisor judges the risk acceptable

C)  The client insists

D)  Never — the call is absolute

15. HPWJ pumps are positive displacement pumps. This means:  (1 mark)

A)  The pump sets the pressure directly

B)  Each stroke pushes a fixed volume, so the pump sets the flow

C)  Pressure and flow are both set at the nozzle

D)  The pump adjusts flow automatically to hold pressure

16. System pressure is created by:  (1 mark)

A)  The pump alone

B)  The hose length

C)  Resistance at the small nozzle orifice

D)  The water temperature

17. Fitting SMALLER plungers to the same pump gives:  (1 mark)

A)  More flow at lower pressure

B)  Less flow but higher available pressure

C)  More flow and more pressure

D)  No change

18. If the nozzle is completely blocked while the pump runs, the pressure:  (1 mark)

A)  Falls to zero

B)  Stays constant

C)  Climbs until the rupture disc bursts or the dump is operated

D)  Oscillates safely

19. The maximum service life of flexible lances and shotgun whip hoses is:  (1 mark)

A)  1 year

B)  2 years

C)  4 years

D)  Unlimited if inspections pass

20. The maximum service life of other high-pressure hoses is:  (1 mark)

A)  2 years

B)  3 years

C)  4 years

D)  5 years

21. In addition to visual inspection, every high-pressure hose requires:  (1 mark)

A)  Monthly flushing

B)  An annual pressure test by a third party

C)  Painting every year

D)  Replacement of fittings every 6 months

22. Which of these is a hose REJECTION criterion?  (1 mark)

A)  Faded outer cover colour

B)  Manufacturer label in a foreign language

C)  Exposed or rusted reinforcement braid

D)  Hose longer than 20 metres

23. Whip checks must be fitted:  (1 mark)

A)  On the pump side only

B)  On the gun side only

C)  On both sides of every high-pressure connection

D)  Only on hoses older than 2 years

24. Quick-connect couplings on high-pressure hoses are:  (1 mark)

A)  Preferred for speed

B)  Allowed below 10,000 psi

C)  Not used — they can rotate and release under pressure

D)  Allowed with a whip check

25. Typical nozzle wear life is:  (1 mark)

A)  5–10 hours

B)  20–60 hours

C)  100–200 hours

D)  Nozzles do not wear

26. The minimum rigid lance length from trigger to tip is:  (1 mark)

A)  0.6 m

B)  0.9 m

C)  1.2 m

D)  2.0 m

27. The visible mark on a lance or hose must be no closer to the nozzle tip than:  (1 mark)

A)  100 mm

B)  300 mm

C)  600 mm

D)  1,000 mm

28. For tube work, the rigid leader (stinger) must be:  (1 mark)

A)  Shorter than the tube

B)  Longer than the pipe's internal diameter

C)  Exactly 600 mm

D)  Made of flexible hose

29. The hard limits for hand-held shotgunning are:  (1 mark)

A)  15,000 psi and 2.0 mm orifice

B)  10,000 psi and 1.6 mm orifice

C)  20,000 psi and 1.0 mm orifice

D)  No limits if reaction force is calculated

30. The anti-withdrawal device (AWD) on a flexible lance:  (1 mark)

A)  Is optional on short tubes

B)  Reduces water consumption

C)  Physically prevents the lance shooting backward out of the tube — mandatory with no exceptions

D)  Is only needed above 20,000 psi

# Section B — Short Answer (10 questions × 2 marks = 20 marks)

31. Name the three roles that make up the minimum HPWJ team.  (2 marks)

_________________________________________________________________________

_________________________________________________________________________

32. Write the imperial reaction-force formula.  (2 marks)

_________________________________________________________________________

_________________________________________________________________________

33. What must the team do the moment communication is lost between the lance operator and pump operator, and why?  (2 marks)

_________________________________________________________________________

_________________________________________________________________________

34. Give two reasons why any jetting injury is treated as a medical emergency until proven otherwise.  (2 marks)

_________________________________________________________________________

_________________________________________________________________________

35. What is the Medical Alert Card, and when must an operator carry it?  (2 marks)

_________________________________________________________________________

_________________________________________________________________________

36. What does SIMOPS mean? Give one example relevant to jetting work.  (2 marks)

_________________________________________________________________________

_________________________________________________________________________

37. Name two of the three most common root causes of serious HPWJ incidents.  (2 marks)

_________________________________________________________________________

_________________________________________________________________________

38. Why must PPE be matched to the job's pressure class?  (2 marks)

_________________________________________________________________________

_________________________________________________________________________

39. What is the FIRST action the team must take the moment any injury occurs during jetting, before anything else — and why?  (2 marks)

_________________________________________________________________________

_________________________________________________________________________

40. What warning does the 600 mm tip mark give the operator, and why does it matter?  (2 marks)

_________________________________________________________________________

_________________________________________________________________________

# Section C — Scenarios (5 questions × 4 marks = 20 marks)

41. A hand-held job is planned at 8 GPM and 12,000 psi. Calculate the approximate reaction force in pounds-force and in newtons (√12,000 ≈ 110; 1 lbf ≈ 4.45 N). State whether the job passes the absolute force limit, and what must still be checked before hand-held work is approved.  (4 marks)

_________________________________________________________________________

_________________________________________________________________________

_________________________________________________________________________

_________________________________________________________________________

42. A metric job is planned at 40 L/min and 1,200 bar. Calculate the approximate reaction force in newtons (√1,200 ≈ 34.6) and state the correct decision.  (4 marks)

_________________________________________________________________________

_________________________________________________________________________

_________________________________________________________________________

_________________________________________________________________________

43. Mid-job, a crane crew begins rigging a lift directly beside your barricaded jetting area. Nobody told your team. Describe the correct response, step by step.  (4 marks)

_________________________________________________________________________

_________________________________________________________________________

_________________________________________________________________________

_________________________________________________________________________

44. During work, a colleague's hand is caught by the jet. The wound looks like a small puncture and he says he feels fine and wants to continue. Describe the correct response, step by step.  (4 marks)

_________________________________________________________________________

_________________________________________________________________________

_________________________________________________________________________

_________________________________________________________________________

45. You arrive at a flexible lance job. The anti-withdrawal device is missing, and the supervisor says the schedule is tight and asks you to run the job anyway. Describe the correct response and justify it.  (4 marks)

_________________________________________________________________________

_________________________________________________________________________

_________________________________________________________________________

_________________________________________________________________________

End of examination. Check that you have answered every question.