OPERATIONS MANAGEMENT SYSTEM

Attachment 8.2

High Pressure Water Jetting (HPWJ)

(Mandatory for KSA and optional/recommended for other Regions)

Table of Contents

PURPOSE	3

REQUIREMENTS	3

2.1	Selecting HPWJ method 	3

2.2	Planning HPWJ Job	5

2.3    General Preparations for HPWJ Job	6

2.4	Site preparation for HPWJ Job 	7

2.5	HPWJ Work Process	10

2.6	Setting up HPWJ equipment and area 	11

2.7    Operation Arrangements ………………………………………………………………………………………..13

2.8	Legal and Regulatory Requirements	14

2.9	Control of Records	14

APPENDIX	15

3.1	Attachment 1:  Pressurise water jetting Technical specification form 	15

3.2	Attachment 2: Maximum Permitted Pressures	15

3.3	Attachment 3:  Hydrojetting Awareness PPT	15

3.4	Attachment 4: (Example) Medical Card	15

3.5	Attachment 5: DO’s and DON’T’s	15

3.6	Attachment 6: Pre-job HPWJ safety checklist	15

3.7    Attachment 7:  HPWJ Work Permit………………………………..….……………………….……..……...15

3.8    Attachment 8: (Example) HPWJ PPE Suit……………………………………………….…………………15

3.9    Attachment 9: HPWJ equipment details ……….………………………………………….....…………15

# PURPOSE

### The purpose of this attachment (a procedure specifically for KSA Sites; mandatory for MEA region and optional / recommended for other Regions), is to define minimum requirements for assessing and controlling the risks and establish safe work system for various types of high-pressure water jetting (HPWJ) working methods. It defines arrangements including qualification requirements, various working methods for high pressure, risks & control measures for safe work as well as equipment care, maintenance & inspection needs.

- Guidance

Selecting safe HPWJ methodology, having competent personnel and adequately tested, inspected & maintained equipment are key elements for a safe HPWJ job.

# REQUIREMENTS (KSA mandatory, Optional / recommended in other regions)

Selecting HPWJ method:

High pressure water jetting (HPWJ) activities are associated with risks and historically many injuries related to HPWJ were reported. Therefore, allowing and selecting method of HPWJ shall be considered with extreme caution.

Considering hierarchy of control, apply safe alternate safe methods instead of HPWJ as far as possible, in consultation with process Engineering and Asset Engineering Department SME’s. Where reasonably practicable, all High Pressure Water Jetting (HPWJ) should be carried out in a designated safe area rather than in situ.

The following are typical hazards & risk considerations for selecting appropriate HPWJ method (see below guidance for arrangement sketch). A specific JSA and/or Risk Assessment considering the HPWJ method, location and equipment shall be used for all HPWJ cleaning jobs with controls/personnel entry inside the jetting area :

| METHOD WITH RISK ( Table 4.1) -Typical for RA guidance only Note: A specific JSA and/or Risk Assessment considering the HPWJ method, location and equipment shall be used for all HPWJ cleaning jobs with controls/personnel entry inside the jetting area Note: Green – Insignificant, Yellow- Minor, Orange – Significant, Red – Major risk |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hazard | Fully automated mechanized cleaning with controls outside the jetting area | Mechanized cleaning with controls outside the jetting area (>6 meters or with physical shielding) | Mechanized cleaning with controls inside the jetting area | Manual cleaning with strain relief (without reaction force) inside the jetting area | Manual cleaning without strain relief (exposed to reaction force) inside the jetting area | Manual cleaning with a flexible lance and a hose catcher system (flexible lance in the hand) | Manual cleaning with a flexible lance without a hose catcher system or with an open-ended hose |
| Injuries in the jetting and working area |  |  |  |  |  |  |  |
| HP water jet | Insignificant | Insignificant | Minor | Minor | Significant | Significant | Major |
| Flying - dirt or HP components | Insignificant | Insignificant | Minor | Significant | Significant | Significant | Significant |
| Parts of the cleaning equipment | Insignificant | Insignificant | Minor | Significant | Significant | Significant | Significant |
| Burns from hot water jets | Insignificant | Insignificant | Minor | Significant | Significant | Significant | Significant |
| Exposure to hazardous substances |  |  |  |  |  |  |  |
| Release of product | Insignificant | Insignificant | Minor | Minor | Minor | Significant | Significant |
| Damage to hearing |  |  |  |  |  |  |  |
| HP system | Insignificant | Minor | Minor | Significant | Significant | Significant | Significant |
| HP components | Insignificant | Minor | Minor | Significant | Significant | Significant | Significant |
| Excessive ergonomic stresses on the body |  |  |  |  |  |  |  |
| Wrong working position | Insignificant | Insignificant | Insignificant | Minor | Significant | Significant | Significant |
| High-frequency vibrations | Insignificant | Insignificant | Insignificant | Minor | Significant | Significant | Significant |
| Overstretching when lifting/absorbing the reaction forces | Insignificant | Insignificant | Insignificant | Minor | Significant | Significant | Significant |
| Excessive thermal load on the body |  |  |  |  |  |  |  |
| Overheating (heat) | Insignificant | Insignificant | Minor | Minor | Significant | Significant | Significant |
| Hypothermia (cold) | Insignificant | Insignificant | Minor | Minor | Minor | Significant | Significant |
| Static electricity |  |  |  |  |  |  |  |
| Fire/explosion | Minor | Minor | Minor | Minor | Minor | Minor | Minor |
| PERFORM HPWJ |  |  |  |  |  |  |  |
| Permission to Perform HPWJ | Yes | Yes | Yes with team JSA | Yes with team JSA | Discouraged, allowed with RA approved by Area sr. Manager | Only in rare cases, allowed with RA approved by Director | Not permitted |
| PPE |  |  |  |  |  |  |  |
| PPE | Basic PPE | Basic PPE | Full suit for the HPWJ pressure when inside jetting area | Full suit for the HPWJ pressure when inside jetting area | Full suit for the HPWJ pressure when inside jetting area | Full suit for the HPWJ pressure when inside jetting area | Not Permitted |

- Guidance

HPWJ methodologies are constantly evolving based on regulations, latest technology as well learning from major incidents. The standard is developed based on available best practices and body of knowledge available like SIR- Manual HP cleaning www.sir-safe.nl

The pressure required to penetrate the skin is of the order of 7 bar ( 100 Psi)

## Planning HPWJ job:

End user Manager e.g. Site Maintenance Manager shall appoint High Pressure Water Jetting HPWJ authorizing personnel, trained to review Pressurized Water Jetting Technical Specifications & hazard controls and supervise HPWJ job.

The High Pressure Water Jetting service shall be selected from the approved list of contractors maintained by SABIC GPS, specialized in the rated pressure and method/technique.

Before starting the job, HPWJ authorizing personnel shall bring alignment with all involved parties and acknowledge understanding & acceptance of the specification and controls identified. Typically, the HPWJ authorizing personnel shall ensure a HPWJ Technical Specification form is agreed upon. (Example: Attachment -1 “HPWJ Technical Specification form”), which shall include:

Definition of the relevant technical aspects of the plant (e.g. materials of construction, condition of item, equipment tags, description of job).

Selected HPWJ method and any unusual hazards posed by the water jetting method to be employed, based on hierarchy of risk control.

The high-pressure water jetting parameters to be used including, water source,  maximum pressure employed, Water flow GPM, any specific equipment/nozzle requirements, location of the job - in situ or in designated area or at contractor facility.

Guidance on maximum HPWJ pressures permitted for various materials are available in Attachment -2 “Maximum permitted HPWJ Pressures”. Approval to exceed these pressures shall be taken from Process and Asset Engineering Managers, if needed.

Any requirements for access, tenting, protective barriers, screening etc. which needs to be arranged to allow the water jetting operation to proceed.

Details of any residual process materials which may present a hazard, based on SDS review.

Definition of any specific environmental controls required considering disposal of water after consultation with site environmental team.

Lone working shall not be allowed while performing HPWJ.

Approved HPWJ Technical Specification form requirements are used to prepare work site, JSA and Work permit.

## General Preparations for HPWJ job:

Conduct a kick-off meeting, as per SHEM 05.00 with the Contractor to communicate all applicable EHSS/general/technical/critical requirements related to the scope of work that the contractor shall comply with.

Ensure all third party employees have received EHSS orientation.

HPWJ employees shall be aware of general as well as site specific & HPWJ task specific EHSS requirements. See sample of Attachment -3 “HPWJ Hazard Awareness”.

HPWJ authorizing personnel shall ensure the HPWJ contractor team members have received the necessary training, certification and qualifications, which shall also be stamped in their third party qualification passport. HPWJ supervisors and operators shall have third party certification & qualification from TQCP or equivalent for operating the proposed HPWJ method/technique (e.g. An operator qualified in automatic HPWJ machines cannot perform manual HPWJ) and have a minimum experience of one year in similar work. Third party certification to be refreshed on two-yearly basis.

HPWJ cutting are specialist techniques and shall only be carried out by competent Operators experienced specifically in these techniques.

All employees are medically fit to perform their assigned work as required by local, regional and national regulations.

Due to the special nature of HPWJ injury, immediate medical attention is required and hence medical aid for first aid and medical treatment of HPWJ injury shall be readily available. Injuries caused by the impact of a water jet may appear insignificant and give little indication of the extent of the injury beneath the skin. To ensure that treatment for HPWJ is not overlooked, all operators engaged in HPWJ tasks should carry an immediately accessible waterproof card that outlines the possible nature of the injury. See sample Attachment-4 “Waterjet Medical Alert Card”.

HPWJ authorizing personnel shall ensure HPWJ machine, its high-pressure components and safety features are inspected annually by third party specialized in HPWJ machine inspection. Contractor shall readily provide documentation, certificates & tags for verification and shall be always at  compliance with applicable local legislation, regulations and manufacturer’s guidelines for safety and fitness of their equipment.

HPWJ authorizing personnel shall coordinate to develop a formal Job Safety Analysis (JSA), as per SHEM 08.10/ SHEM 02.02, in cooperation with the HPWJ contractor supervisor, area owner and safety representative to ensure all mitigation actions, including “line of fire” and PPE required are identified. Team shall use Table 4.1 (but not limited to) as a basis for HPWJ hazards.

HPWJ machine shall be inspected prior entering to the facility and are labeled by inspection sticker by site-authorized inspector.

## KSA Site preparations for High Pressure Water Jetting (HPWJ):

HPWJ activity site shall be prepared as per requirements in  Attachment -1 “HPWJ Technical Specification form”

Water supply source and water-jetting effluent disposal:

Select source (potable quality and free of suspended solids) to provide adequate HPWJ water supply even at maximum desired pressure. Ensure the filter is clean.

Connection to fire equipment ( e.g. hydrant) as water supply source shall follow SHEM 11.04 “Emergency Response Equipment Interruption & Authorized Use of Emergency Equipment for Non-Emergency Purpose”.

Provide provision for adequate drainage to prevent build-up of water. Provisions shall be made to ensure that water-jetting effluent does not contaminate, block, damage or overload the drains. If chemicals (e.g. toxicity, potential environmental impact) are involved, arrange disposal of water after consultation with site environmental team.

Jetting area Precautions:

Wherever possible, HPWJ should be carried out in ‘special cleaning areas’, which are   quiet rarely-visited areas.   The areas shall be suitably enclosed and “No Entry” signs prominently displayed at access points and perimeters.  All HPWJ controls are applicable.

The limits of the area in which work is permissible considering the danger of the jet striking adjacent or hidden cables, obstructions, etc.  and any tenting/barriering/screening requirements specified on the Technical Specification form.

HPWJ shall not be carried out on pebbled areas.

Provide adequate lighting where HPWJ is carried out during hours of darkness or in enclosed dim lit areas.

Consider alternatives to manual water jetting in confined spaces, which should be avoided if reasonably practicable. If water jetting is required in a confined space then consideration must be given to the increased likelihood of operator being struck by ricocheting jet or by dislodged debris, the reduced vision caused by rapid build-up of spray/mist, footing being affected by slippery surfaces.  An appropriate confined space entry work permit shall be issued for entering as per SHEM 08.10.

Electrical Precautions:

Every effort should be made to remove the flammable material from service equipment prior to water jetting, and other cleaning techniques considered as an alternative.

If service equipment contains flammable material, evaluate the need for extra precautions to be taken to dissipate static charge accumulations, which may occur as a result of using a water jet.    If flammable material remains or may be disturbed by the water jetting then suitable grounding/bonding shall be provided on the jetting machine and the continuity established from the jetting nozzle to the machine as per  SHEM 08.05 “Electrical Safety”.

High pressure water jetting equipment uses a drive ( e.g.diesel engine) and electrical/control equipment, which shall consider hazardous area classification and work permit requirements.

Tools and lamps used shall follow hazardous area classification and work permit requirements

Provide protection for nearby sensitive equipment ( e.g. electrical /instrument panels) from water.

Safe work platform:

Provide escape routes for all HPWJ operators, which shall not be blocked with hoses, machines, protective sheets or barriers.

Safe working platform including safe footing considering back thrust shall be provided. The reaction force of water jet can either act directly on the HPWJ operator or the structure from which work is carried out causing the operator to lose his footing and fall. The maximum reaction force shall be no more than 250 Newtons in the longitudinal direction. Note: The force produced by a high-pressure water gun and the amount of force the operator must withstand is calculated as follows:

- Back Thrust (Ibf.) = 0.052 x flow rate x pressure

- (Flow in GPM, Pressure in Psi; 1 lbf =4.448N)

No water jetting shall ever be carried out from a ladder. All scaffolding used shall be designed, erected and managed by competent persons and adequately secured to prevent it being pushed over . Fall arrest devices / full body harness shall be used following SHEM 08.09 “Working at height”

Proper work-rest regimen shall be followed to avoid fatigue and use of non-slip footwear helps to prevent slips & falls.

The work area shall be clear of all loose items or debris to avoid trips and falls.

Where practicable, steel guarding shall be provided for the lance operator to stand behind.  Where jetting requires the lance to be pointed downwards, (e.g. when cleaning a tube bundle), this shall be done from a platform above the face of the work piece.  The operator shall stand behind a steel guard to protect legs and feet from the water jet.

Personal Protective Equipment (PPE): PPE guidance for HPWJ

Consider PPE as the last resort in hierarchy of risk control and only use PPE conforming to industry standards and SHEM 08.08 “PPE”, as per JSA assessments. PPE users shall be trained in the proper use and handling of PPE.

Head and eye:  Hard hat, face shield & safety glasses.  Head protection 20/30; If there are hazards from aerosols, fine sprays or dusts, goggles shall be worn in addition to the face shield. If a waterproof hood is used this should fit over the helmet.

Hearing Protection:  As required for noise level (ear plugs, ear muffs or both); Hearing protection necessary in noisy plant areas or when working with or adjacent to the water jetting operation

Body Protection: 10/28 up to 14,500psi; 20/30 up to 30,000psi  or TurtleSkin WaterArmor ;

Hand Protection:  10/28 up to 14,500psi; 20/30 up to 30,000psi  or TurtleSkin WaterArmor;

Foot Protection:  Waterproof boots with Safety (steel or Kevlar) Toes and Metatarsal Guards. For high Pressures,  3000 boots or TurtleSkin WaterArmor;

Gun Operators shall also have shin guards.

Full face visor (to BS EN 166 Impact Grade B as a minimum standard) with chin protection

Anti-slippery, water proof gloves. Care should be taken to ensure gloves are the correct size and that the HPWJ tasks can be conducted when wearing the gloves e.g. Gun trigger can jam when too large gloves are used. Specialist hand protectors with enhanced resistance to jet penetration are recommended.

If the nature of the material likely to be released during jetting is such that it is possible that the above items will not provide adequate protection (e.g. it is toxic or corrosive), then additional clothing such as a PVC hood or Respiratory Protective Equipment shall be specified - based on a MSDS/assessment of the material and activity.

If HPWJ generates hazardous material mist/aerosol, correctly selected respiratory protective equipment (RPE) suitable for use with other head-mounted PPE shall be provided. RPE users shall be trained for use, maintenance and disposal, as per SHEM 08.08.

Full body protection suit for the HPWJ pressure. See attachment 8 (Example) PPE HPWJ Suit.

## HPWJ  Work process (KSA mandatory, Optional / recommended in other regions)

HPWJ authorizing personnel shall ensure a dedicated formal Job Safety Analysis (JSA) for the high pressure water jetting activity defining the risk controls and techniques/safe methods is approved and available.  JSA together with the technical specification, shall be made available to the work permit issuer for preparing work permit, as per Table 4.1 guidelines (but not limited to) as a basis for HPWJ hazards.

Acceptance of permits for high pressure water jetting activities shall only be undertaken by the approved Water Jetting Supervisor.

Before work starts, the HPWJ authorizing personnel will review the JSA. HPWJ method and controls at the work location with the HPWJ team and each team member including the Supervisor will endorse this to demonstrate understanding of its requirements.

If changes or deviations arise during the work, the HPWJ Supervisor shall stop the activity immediately and notify the Work Permit Issuer and approved HPWJ authorizing personnel.

The following are the overview of HPWJ work process:

| - STEP | - RESPONSIBILITY |
| --- | --- |
| - Assess alternative cleaning methods and if HPWJ selected, complete Technical form ATT-1 | - Job requester and HPWJ authorizing person (E.g. Maintenance Supervisor) - Note: Apply safe alternate safe methods instead of HPWJ as far as possible, in consultation with process Engineering and Asset Engineering Department SME’s. |
| - Arrange/plan for HPWJ work with approved contractor | - HPWJ authorizing person |
| - Off-site Contractor preparation (equipment, personnel, PPE, inspection, training, transport,) | - HPWJ Contractor Supervisor & employees |
| - Prepare formal JSA/RA (Refer Table 4.1 guidelines), inspect equipment, Prepare for HPWJ set-up at site. - Complete work permit request | - HPWJ authorizing person with Contractor Supervisor & Safety team |
| - Review permit request and issue permit to set up site | - Site work permit issuer |
| - Set up site | - HPWJ Contractor Supervisor & Safety team agreed by HPWJ authorizing person and area owner |
| - Review Field set up and fill up ATT-6 Pre-job HPWJ checklist - Issue HPWJ work permit | - Work permit Issuer with support of HPWJ authorizing person and Contractor Supervisor |
| - Perform HPWJ work complying to safe work practices & site instructions | - HPWJ Contractor Supervisor & employees - HPWJ authorizing person to supervise |
| - Remove equipment, restore site | - HPWJ Contractor Supervisor & employees |
| - Return and sign-off permit | - HPWJ Contractor Supervisor & Safety team agreed by HPWJ authorizing person and area owner |

## Setting up High Pressure Water Jetting (HPWJ) Equipment and Area (KSA mandatory, Optional / recommended in other regions)

High Pressure Water Jetting (HPWJ) contractor shall provide the necessary high pressure water pumps, hoses, lances, and associated equipment including failsafe dump (dead mans) control valves, safety features to allow the work detailed on the HPWJ Method and Technical Specification to be carried out.  The HPWJ contractor shall ensure that all such equipment is maintained and serviced in accordance with the legal requirements, manufacturer’s guidelines and best practices.

For high pressure water pump units the pressure control regulator shall be via a control knob with an arrow head and calibrated indicator markings or a calibrated gauge, with pressure level readout close to the control valve.  All such equipment should be calibrated at least annually.

High pressure hoses shall not have kinks, cuts or chipping on outer rubber.  Hoses shall also have the pressure rating engraved on its couplings or hose body. High pressure hoses shall be pressure tested, once every year by a competent authority. Records of all such test shall be kept.

The HPWJ supervisor shall ensure that hoses used are rated for the duty and that couplings used are suitable for the pressure.  Particular care should be taken in laying out equipment to ensure that it is undamaged and not positioned where damage is likely to be sustained. Each hose is visually inspected at each use and damaged hoses are replaced. Splicing hoses is not acceptable – lengths of hose will only be joined by an appropriate fitting.

Design and maintenance of lances and nozzles shall conform to the requirements or industry best practices.  In particular, lances shall be a minimum of 1.2m in length.  If shorter lances have to be used they shall have a double trigger mechanism and include additional hand protection on the lance.  Flexible hoses must also have a stainless steel rigid lance/leader of at least 300mm fitted to the nozzle end of the hose.

The HPWJ Supervisor shall, when permitted by the Work Control Permit Issuer, ensure set up of the equipment in accordance with Technical form, JSA and best practices taking into account any additional requirements from the Permit Issuer.

The HPWJ Supervisor will agree with the WCP Issuer the best means of routing and protecting hoses.  Where practicable hoses should not be routed across roadways or other areas where they could be damaged.  If this is unavoidable then they shall be protected from damage with suitable ramps/shrouds/conduits, and warning notices shall be displayed.  Hoses should be routed well away from pedestrian pathways or walkways.  Consideration should be given to securing hoses at regular intervals when being run up high structures including the fitting of hose restraining devices to protect against coupling failure and to the shielding of hoses with covers or conduits.

All daily and pre-use checks shall be carried out by the HPWJ Supervisor before any water jetting Operation using Attachment 6 “Pre-job HPWJ safety checklist”. This is pre-requisite to apply for Attachment 7 “HPWJ work permit”.

Electrically powered equipment shall follow standards of design, control and installation to avoid hazards of electrocution.

The HPWJ Supervisor shall consider whether any additional precautions are required as a consequence of any chemicals or additives being used. When HPWJ equipment is used to remove coatings or to clean surfaces that are contaminated with a hazardous substance or bacteria, it is possible these may be entrained in the resulting aerosol.

The Water Jetting Supervisor, in consultation with the WCP Issuer shall ensure provision of appropriate barriering and shielding to prevent access to the activity and to protect adjacent equipment against the water jetting operation.  Barriers shall be rope or tape, as a minimum, and be positioned not less than 6 meters away from the work piece.  Alternatively robust protective screens (like steel, wood etc) at least 2 metres high able to resist a high velocity jet or flying debris are acceptable at a closer distance (but not closer than 3 meters from the jet), such controls would have to be agreed between the operator and process supervisor and included in the risk controls on the Work Control Permit.  Danger notices shall also be displayed at all approaches to the working area and shall state “NO UNAUTHORISED ENTRY - HIGH PRESSURE WATER JETTING IN PROGRESS”.

Refer Attachment 9: “ HPWJ equipment details” for details about HPWJ equipment and accessories.

## Operational Arrangements (KSA mandatory, Optional / recommended in other regions)

The Work Control Permit Issuer shall only allow issue of the Work Control Permit after personally checking that all warning notices and other protective measures are in place and when the HPWJ Technical Specification has been attached to the Work Control Permit.

The HPWJ Supervisor shall ensure that his team consists of sufficient individuals such that messages can be passed between the lance holder and the compressor operator without the need to move round corners or other blind spots. Communication methods shall be agreed upon before work starts.

There shall be two members as minimum in HPWJ team.  One member shall operate the lance and has the primary duty of directing the jet stream.  The second member shall attend the pump unit, keep close watch on the first member for signs of difficulty or fatigue, and monitor the surrounding area for intrusion by other persons or unsafe situations.

Further members may be required in the following circumstances: - to operate the dump valve (dead man’s handle) if it is remote from the operative controlling the jet, to assist in controlling the lance, when working in a fully sheeted out enclosure or to retain contact between the operative controlling the lance and the operative controlling the pump unit.

All HPWJ activities shall be controlled by a trained and competent HPWJ supervisor, who may be one of the members undertaking the activity.  The HPWJ supervisor is responsible for ensuring the basic equipment checks have been undertaken and everything is in place to allow the work to proceed safely.

The member controlling the lance shall take the lead role whilst jetting is in progress, however the team members may rotate their duties during any job to minimize fatigue.

The controls will be further validated and endorsed by the approved and appointed Water Jetting Supervisor who will be the primary liaison with the WCP Issuer.

The HPWJ authorizing person will undertake periodic visits to the task to ensure the required controls and job method are being adhered to.

Only the HPWJ team members are permitted in the barriered area while jetting is in progress.  In case of inspection or entry into jetting area, HPWJ shall be suspended and HPWJ Supervisor shall accompany the entrant.

The basic safety features to allow HPWJ to be performed safely, but not limited to, are listed in Attachment 5: “HIGH PRESSURISE WATER JETTING (HPWJ) DO’s and DONT’s”

## Legal and Regulatory Requirements (KSA mandatory, Optional / recommended in other regions)

Each SABIC Division/Entity in KSA shall ensure that all applicable local, national, and state/regional regulations and requirements and all international standards and treaties to which each SABIC Division/Entity subscribes are identified and incorporated into the relevant Level 3 management system documents in accordance with Compliance, Assessments and Evaluation standard (OMS-232).

Compliance assessments shall be performed by knowledgeable and competent people at planned intervals.

Guidance

Resources that can be used to determine the applicable local/national/regional/international regulations and standards related to HPWJ include (but are not limited to) ASTM E1575 – 18 Standard Practice for Pressure Water Cleaning and Cutting, Code of Practice for the use of High Pressure and Ultra High Pressure Water Jetting Equipment– The Water Jetting Association (UK, Dutch etc.)

The assessment process and interval should be discussed and aligned with the OMS-SHEM-08.00 Element Leader to align the approach between all OMS-SHEM-08 Sub-Elements.

## Control of Records (KSA mandatory, Optional / recommended in other regions)

Each SABIC Division/Entity in KSA shall identify and describe the record control requirements pertaining to this attachment in accordance with Documentation & Control of Records Standard (OMS-234).

Ensure alignment with Documentation & Control of Records (OMS-234)

Periodic and accurate Inspection records for all HPWJ equipment, accessories, tools and tackles shall be maintained.

For the operation of HPWJ equipment, if there is no manufacturer's specifications and limitations are available, proper document and record shall be maintained regarding the limitations assigned to the equipment based on the determinations of a qualified engineer competent in this field.

# APPENDIX

## Attachment 1: HPWJ Technical Specification form

http://ecm.sabic.com/ecm/llisapi.dll/link/624113899

## Attachment 2: Maximum Permitted HPWJ Pressures

http://ecm.sabic.com/ecm/llisapi.dll/link/624130421

## Attachment 3: HPWJ Awareness

http://ecm.sabic.com/ecm/llisapi.dll/link/624113898

## Attachment 4: HPWJ Medical Alert Card Sample

http://ecm.sabic.com/ecm/llisapi.dll/link/624119339

## Attachment 5: HIGH PRESSURISE WATER JETTING (HPWJ) DO’s and DONT’s

http://ecm.sabic.com/ecm/llisapi.dll/link/624113897

## Attachment 6: Pre-Job HPWJ safety checklist

http://ecm.sabic.com/ecm/llisapi.dll/link/624128784

## Attachment 7: HPWJ workpermit

Use SHEM 08.10/OMS 316.10 H Appendix as HPWJ workpermit

App# 6 is a pre-requisite for issuing HPWJ workpermit.

## Attachment 8: HPWJ PPE example suit

http://ecm.sabic.com/ecm/llisapi.dll/link/624128306

## Attachment 9: HPWJ Equipment details

http://ecm.sabic.com/ecm/llisapi.dll/link/624112182